## UNIT – 2 (NOTES)

- 1.Major sources of water pollution:

## Sources Of Water Pollution

The key causative of water pollution in India are:

- Urbanization.

- Deforestation.

- Industrial effluents.

- Social and Religious Practices.

- Use of Detergents and Fertilizers.

- Agricultural run-offs- Use of insecticides and pesticides.

### Water Pollution – A Modern Epidemic

One of the primary causes of water pollution is the contamination of water bodies by toxic chemicals. As seen in the example mentioned above, the dumped plastic bottles, tins, water cans and other wastes pollute the water bodies. These result in water pollution, which harms not just humans, but the whole ecosystem. Toxins drained from these pollutants, travel up to the food chain and eventually affect humans. In most cases, the outcome is destructive to only the local population and species, but it can have an impact on a global scale too.

Nearly 6 billion kilograms of garbage is dumped every year in the oceans. Apart from industrial effluents and untreated sewage, other forms of unwanted materials are dumped into various water bodies. These can range from nuclear waste to oil spills – the latter of which can render vast areas uninhabitable.

- 1. Eutrophication

One of the most severe and commonest water pollution problems is due to enrichment of waters by plant nutrients that increases the biological growth and renders the water bodies unfit for diverse uses. The process of increase in the nutrients of waters and resultant spurt in algal productivity is called eutrophication.

The term eutrophication has been derived from a Greek word eutrophos meaning corpulent or rich. The first use of this term in ecology was made in connection with the remnants of the extinct lakes rather than with live lakes. Weber (1907), while studying the evolution of North German peat bogs, found that the upper layers have more nutrients in comparison to the lower ones as the original lakes received much higher nutrient supply prior to their transformation into bogs. He used the terms eutrophic (rich in nutrients) and oligotrophic (poor in nutrients) to distinguish between these two layers. The use of these terms in limnology was made for the first time by Naumann (1919), in order to denote nutrient poor (oligotrophic) and nutrient rich (eutrophic) conditions in relation to the development of different algal associations.

- The Process Of Eutrophication
The eutrophication is basically a natural phenomenon which gets accelerated by increased nutrient supply through human activities. The process of eutrophication starts as soon as the lakes are formed because of the entry of nutrients by natural means, but the rate of eutrophication remains quite low under natural conditions. The process of eutrophication can be discussed under two heads of natural and accelerated processes, though its basic features remain essentially the same.
1. Natural eutrophication
The lakes generally originate as oligotrophic and have only limited quantities of nutrients depending upon the mode of their formation and composition of original sediments. These nutrients are insufficient to produce any significant algal growth. At this stage the lakes have only autochthonous nutrients (indigenous nutrients cycling therein), which usually recycle completely in the absence of any outside supply. All the biological production is completely decomposed after death. As the allochthonous nutrients (nutrients from outside) start entering the lake, the process of eutrophication sets in. The principal natural sources of nutrients are the natural run-off, fall of leaves and twigs from the surrounding vegetation, periodical submergence of the nearby terrestrial vegetation, rain fall and bird droppings etc.
The build-up of nutrients through this slow mode of entry gradually starts increasing the growth of algae. When the algae die and decompose, the locked nutrients are again made available to the fresh algal growth. The tropical or hot climate usually supports a higher rate of eutrophication as it favours higher nutrients utilization and algal growth in comparison to cold and temperate climates.
2. Accelerated Eutrophication
The process of eutrophication is greatly augmented by the increased supply of nutrients through various human activities such as discharge of domestic sewage, industrial wasters, agricultural and urban run-off. Increased levels of air pollution also make the water bodies rich in nutrients through their transport with rains or by dry fallout. This increased supply of nutrients triggers the algal growth at much faster rate, thus increasing the speed of eutrophication, which otherwise would have been a slow natural phenomenon. The process of eutrophication is, therefore, sometimes referred to as ageing of lakes.

- 2. Acid mine drains :-

remains one of the most significant challenges in the mining industry, whether in gold or coal operations. It forms when rocks containing sulphide minerals react with rainwater and air, producing acidic water capable of dissolving metals. When this water flows into rivers or lakes, it can damage ecosystems and pose risks to living organisms.

Traditionally, mining companies have treated acid water by adding lime or other chemicals to neutralise its pH. While this method works quickly, it is costly and generates large volumes of sludge that require further handling. For this reason, researchers are increasingly exploring more natural and sustainable alternatives: bioremediation.

- Bioremediation makes use of microorganisms that can convert sulphate into sulphide. The sulphide then reacts with dissolved metals to form harmless precipitates, resulting in cleaner water with a near-neutral pH. These microbes thrive by consuming simple organic materials such as straw or compost as their energy source.

- In several countries, constructed wetlands have already been established to treat acid mine drainage using plants and bacteria. Such systems not only neutralise water but also create green habitats for birds and insects. Another approach involves simple organic reactors, through which acidic water is channelled across organic matter so that microbes can work more effectively.

- 3.  Pesticides

The term pesticide is used to cover a range of chemicals that kill organisms that humans consider undesirable and includes the more specific categories of insecticides, herbicides, rodenticides, and fungicides.  There are three main groups of synthetic organic insecticides: organochlorines (also known as chlorinated hydrocarbons), organophosphates, and carbamates.  In addition, a number of herbicides, including the chlorophenoxy compounds 2,4,5-T (which contains the impurity dioxin, which is one of the most potent toxins known) and 2,4-D are common water pollutants.

The most well-known organ chlorine pesticide is DDT (dichlorodiphenyltrichloroethane) which has been widely used to control insects that carry diseases such as malaria, typhus, and plague.  By contributing to the control of these diseases, DDT is credited with saving literally millions of lives worldwide.  In spite of its more recent reputation as a dangerous pesticide, in terms of human toxicity DDT is considered to be relatively safe.  It was its impact on food chains, rather than human toxicity that led to its ban.  Organo chlorine pesticides, such as DDT, have two properties that cause them to be particularly disruptive to food chains.  They are very persistent, which means they last a long time in the environment before being broken down into other substances, and they are quite soluble in lipids, which means they easily accumulate in fatty tissue.  This phenomenon in which the concentration of a chemical increases at higher levels in the food chain is known as biomagnification or bioconcentration.

# 4. DYEING AND TANNING

## 1. Meaning of Dyeing

Dyeing is the process of adding color to textile materials such as fibers, yarns, or fabrics using dyes.

### Objectives of Dyeing

- To improve appearance and attractiveness

- To increase market value of textiles

- To provide color fastness (resistance to washing, light, etc.)

### Types of Dyes

- Natural Dyes

  - Obtained from plants, animals, minerals

  - Example: Indigo, turmeric

  - Eco-friendly but less durable

- Synthetic Dyes

  - Chemically manufactured

  - Bright colors and long-lasting

  - Widely used in industries

## 2. Process of Dyeing

- Preparation

  - Cleaning (removal of dirt, oil, grease)

- Dye Application

  - Fabric immersed in dye solution

- Fixation

  - Dye is fixed using chemicals/heat

- Washing & Finishing

  - Removal of excess dye and drying

## 3. Environmental Impact of Dyeing

- Water pollution due to chemical discharge

- Toxic substances affecting aquatic life

- High water consumption

- Air pollution from chemical fumes

## 4. Meaning of Tanning

Tanning is the process of converting raw animal hides/skins into leather using chemicals.

### Objectives of Tanning

- To make leather durable and flexible

- Prevent decomposition of hides

- Improve texture and usability

## 5. Types of Tanning

- Vegetable Tanning

  - Uses natural tannins (tree bark, leaves)

  - Eco-friendly but slow process

- Chrome Tanning

  - Uses chromium salts

  - Faster and widely used

  - Produces soft and durable leather

## 6. Process of Tanning

- Curing

  - Preservation of raw hides

- Soaking

  - Cleaning and rehydration

- Liming

  - Removal of hair and fats

- Tanning

  - Treatment with tanning agents

- Finishing

  - Coloring, polishing, drying

## 7. Environmental Impact of Tanning

- Release of toxic chemicals (chromium)

- Water and soil pollution

- Health hazards to workers

- Bad odor and air pollution

5. Marine pollution

It is defined as the discharge of waste substances into the sea, posing threat to living sources, hazard to human health, hindrance to fishery and impairment of quality of sea water. Marine pollution is associated with the change in physical, chemical and biological conditions of the sea water.  Nearly 71% of Earth surfaces is covered with Oceans, which comprise a total of approximately 1.37 x 1039 litres. Ocean is an ideal place to dump all the man wastes.

Marine pollutants in the sea.

- Pathogens

- Sediments

- Solid wastes

- Heat

- Freshwater

- Brine

- Toxic Inorganics

- Toxic Orgnics

- Pertoleum and oil

- Nutrients

- Radioactive materials

- Oxygen demanding materials

- Acids and Bases

These pollutants comes from various sources. The Marine pollution may also off natural origin.

Sources of pollutants

- Marine commerce

- Industry

- Electircal Power generation

- Sewage treatment

- Other Non Industrial Wastes

- Recreation

- Construction

- Types of Microplastics

Microplastics are broadly classified into two categories:

### a) Primary Microplastics

These are intentionally manufactured small plastics:

- Microbeads in cosmetics (face scrubs, toothpaste)

- Industrial abrasives

- Plastic pellets (used in manufacturing)

### b) Secondary Microplastics

These are formed by the breakdown of larger plastic items due to:

- Sunlight (UV radiation)

- Physical wear (waves, friction)

- Chemical degradation
Examples:

- Broken plastic bottles

- Degraded plastic bags

- Synthetic fibers from clothes

- Sources of Microplastics

- Household sources: Washing synthetic clothes (polyester, nylon)

- Personal care products: Scrubs, exfoliants

- Tyre wear particles: From vehicles

- Plastic waste degradation: Landfills, oceans

- Industrial discharge

- 2.Techniques to control water pollution:

- sewer system

The conventional gravity sewer system is designed with many branches. Typically, the network is subdivided into primary (main sewer lines along main roads), secondary and tertiary networks (networks at the neighbourhood and household level). This system is mostly found in urban areas.

There are basically three types of sewer lines for conventional gravity sewers: primary, secondary and tertiary networks. The main line runs through the centre of the system, and all lines empty into it. It then carries wastewater to a semi-centralised or centralised treatment facility. Main lines generally have a diameter of 200 to 300 mm, and branch lines extend from them much like branches from the trunk of a tree. The sewage from one or more buildings are house laterals, usually of 100 mm in diameter, and empties into a branch line which usually also has a diameter of 100 mm. All sewer pipes are laid out in straight lines whenever possible and generally meet at right angles although the connection may be curved to ease the flow.

- Waste water treatment

These treatment methods are grouped into three general categories:  primary treatment, secondary or biological treatment and tertiary or advanced treatment.

- Primary Treatment

Untreated or raw wastewater usually flows by gravity from an interceptor or trunk sewer into the head works of a treatment facility; sometimes wastewater may be pumped to the treatment plant in a force.  The head works of a treatment plant include a flow measurement device and mechanical systems that provide preliminary treatment.  Preliminary treatment systems typically include screens, comminutors, and grit chambers.

The first treatment process for raw wastewater is coarse screening.  Bar screens (or racks), as they are called, are made of long, narrow metal bars spaced about 25 mm (1 in.) apart.  They retain floating debris, such as wood rags, or other bulky objects, that could clog pipes or damage mechanical equipment in the rest of the plant.

In some treatment plants, a mechanical cutting or shredding device, called a comminutor, is installed just after the coarse screens.   The comminutor  shreds and chops solids or rags that passed through the bar screen.  The shredded material is removed from the waste water by sedimentation or flotation later in the treatment plant.

- Secondary (Biological) Treatment
Primary treatment processes remove only those pollutants that will either float or settle out by gravity, but about half of the raw pollutant load still remains in the primary effluent.  The purpose of secondary treatment is to remove the suspended solids that did not settle out in the primary tanks and the dissolved BOD that is  unaffected by physical treatment.  Secondary treatment is generally considered to meet 85 per cent BOD and TSS removal efficiency and represents the minimum degree of treatment required in most cases.

Biological treatment of sewage involves the use of microorganisms. The microbes, including bacteria and protozoa, consume the organic pollutants as food.  They metabolize the biodegradable organics, converting them into carbon dioxide, water and energy for their growth and reproduction. A biological sewage treatment system must provide the microorganisms with a comfortable home.  In effect, the treatment plant allows the microbes to stabilize the organic pollutants in a controlled, artificial environment of steel and concrete, rather than in a stream or lake.  This helps to protect the dissolved oxygen balance of the natural aquatic environment.

To keep the microbes happy and productive in their task of wastewater treatment, they must be provided with enough oxygen, adequate contact with the organic material in the sewage, suitable temperatures, and other favourable conditions.  The design and operation of a secondary treatment plant is accomplished with these factors in mind.

Two of the most common biological treatment systems are the trickling filter and the activated sludge process.  The trickling filter is a type of fixed growth system.  The microbes remain fixed or attached to a surface while the wastewater flows over that surface to provide contact with the organics.  Activated sludge is characterized as a suspended – growth system, because the microbes are thoroughly mixed and suspended in the waste water rather than attached to a particular surface.

- Tertiary (Advanced) Treatment
Secondary treatment can remove between 85 and 95 per cent of the BOD and TSS in raw sanitary sewage.  Generally, this leaves 30 mg / L or less of BOD and TSS in the secondary effluent.   But sometimes this level of sewage treatment is not sufficient to protect the aquatic environment.

Another limitation of secondary treatment is that it does not significantly reduce the effluent concentrations of nitrogen and phosphorous in the sewage.   Nitrogen and phosphorous are important plant nutrients.   If they are discharged into a lake, algal blooms and accelerated lake aging or cultural eutrophication may be the result.   Also, the nitrogen in the sewage effluent may be present mostly in the form of ammonia compounds.  These compounds are toxic to fish if the concentrations are high enough.  Yet another problem with the ammonia is that it exerts a nitrogenous oxygen demand in the receiving water as it is converted to nitrates.  This process is called nitrification.
When pollutant removal greater than that provided by secondary treatment is required, either to further reduce the BOD or TSS concentrations in the effluent or to remove plant nutrients, additional or advanced treatment steps are required.  This is also called tertiary treatment, because many of the additional processes follow the primary and secondary processes in sequence.
Tertiary treatment of sewage can remove more than 99 per cent of the pollutants from raw sewage and can produce an effluent of almost drinking water quality.

- Effluent polishing

The removal of additional BOD and TSS from secondary effluents is sometimes referred to as effluent polishing. It is most often accomplished using a granular media filter much like the filters used to purify drinking water.  Since the suspended solids consist mostly of organic compounds, filtration removes BOD as well as TSS.

- Phosphorus Removal
When stream or effluent standards require lower phosphorous concentrations, a tertiary treatment process must be added to the treatment plant.  This usually involves chemical precipitation of the phosphate ions and coagulation.  The organic phosphorous compounds are entrapped in the coagulant flocs that are formed and settle out in a clarifier.

One chemical frequently used in this process is aluminium sulfate ( Al2SO4).  This is called alum, the same coagulant chemical used to purify drinking water.  The aluminium ions in the alum react with the phosphate ions in the sewage to form the insoluble precipitate called aluminium phosphate.  Other coagulant chemicals that may be used to precipitate the phosphorous include ferric chloride (FeCl3), and lime( CaO).

- Aerobic waste water treatment method

Trickling filters
A trickling filter consists basically of a layer or bed of crushed rock about 2m
(6ft) deep.  It is usually circular in shape and may be built as large as 60 m (200 ft) in diameter.   Trickling filters are always preceded by primary treatment to remove coarse and settleable solids.   The primary effluent is sprayed over the surface of the crushed stone bed and trickles downward through the bed to an under drain system.

A rotary distributor arm with nozzles located along its length is usually used to spray the sewage, although sometimes fixed nozzles are used.  The rotary distributor arm is mounted on a center column in the trickling filter; it is driven around by the reaction force or jet action of the waste water that flows through the nozzles.

The under drain system serves to collect and carry away the wastewater from the bottom of the bed and to permit air circulation upward through the stones.  As long as topography permits, the sewage flows from the primary tank to the trickling filter by the force of gravity, rather than by pumping. As the primary effluent trickles downward through the bed of stones, a biological slime of microbes develops on the surfaces of the rocks. The continuing flow of the wastewater over these fixed biological growths provides the needed contact between the microbes and the organics.  The microbes in the thin slime layer absorb the dissolved organics, thus removing oxygen – demanding substances from the waste – water.  Air circulating through the void spaces in the bed of stones provides the needed oxygen for stabilization of the organics by the microbes.

The stones are usually about 75 mm (3 in.) in size, much too large to  filter out suspended solids.  The stones in a trickling filter only serve to provide a large amount of surface area for the biological growths, and the large voids allow ample air circulation. The trickling filter effluent is collected in the under drain system and then conveyed to a sedimentation tank called a secondary clarifier. The secondary clarifier, or final clarifier as it is sometimes called, is similar in most respects to the primary clarifier, although there are differences in detention time, over flow rate, and other details.

To maintain a relatively uniform flow rate thorough the trickling filter and to keep  the distributor arm rotating even during  periods of low sewage flow, some of the waste water may be recirculated.  In other words, a portion of the effluent is pumped back to the trickling filter inlet so that it will pass through the bed of stones more than once. Recirculation can also serve to improve the pollutant removal efficiency; it allows the microbes to remove organics that flowed by them during the previous pass through the bed.

b. Anaerobic wastewater treatment methods

The generation and disposal of large quantities of biodegradable waste without adequate treatment result in widespread environmental pollution. Some waste streams can be treated by conventional methods like aeration. Compared to the aerobic method, anaerobic digestion proves to be more advantageous in terms of efficiency of treatment as well as potential energy savings. Biomethanation is the process of conversion of organic matter in the waste (liquid or solid) to biogas and manure by microbial action in the absence of air. Methane produced by methanogenic bacteria is also another potential energy source. Methane is used for generation of mechanical, heat and electrical energy. Anaerobic decomposition of waste materials produces large amounts of methane. Many sewage treatment plants produce this fuel. Efficient generation of methane can be achieved by using algal biomass grown in pond cultures, sewage sludge, municipal refuse, plant residue and animal waste. Methanogens (Archaebacteria) are obligate anaerobes and produce CH4 by reducing acetate and/or CO2. Biogas, a mixture of different gases is produced by anaerobic microbes using domestic and agricultural wastes. Bulk (about 50 – 70%) of biogas is methane (CH4) and other gases are in low proportions. These include CO2 (25 – 35%), H2 (1 – 5%), N2 (2 – 7%) and O2 (0 – 0.1%). In India a large number of gobar gas plants are already in operation in rural areas. Left overs of these plants are good fertilizers also. Animal waste is first hydrolyzed by hydrolytic bacteria. It is followed by acid formation by a group of acetogenic bacteria, which convert monomers into simple compounds like NH3, CO2 and H2. Finally methanogens reduce acetate and/or CO2 to CH4. In India, cattle dung is the chief source of biogas.

Biomethanation requires adequate infrastructural facilities. The first and the foremost among them is the bioreactor in which the treatment is to be carried out,  since extremely large volumes of effluents are encountered for treatment. Thus, an optimally designed bioreactor can decrease the treatment time and increase the treatment efficiency leading to an overall lowering of the treatment cost. Selection and design of bioreactors are dictated by process kinetics. Conventional digesters such as sludge digesters and anaerobic CSTR (Continuous Stirred Tank Reactor) have been used for many decades in sewage treatment plants for stabilizing the activated sludge and sewage solids. Interest in biomethanation as an energy-saving waste treatment has led to the development of a range of anaerobic reactor designs. These high-rate digesters are also known as retained biomass reactors since they are based on the concept of retaining viable biomass by sludge immobilization.

Anaerobic reactors for liquid waste

- Upflow anaerobic sludge blanket

- Anaerobic fluidized bed

- Anaerobic filter

- Expanded granular sludge bed reactor

The dairy industry generates a large volume of wastewater, which is generated both in the production of the products and in the packaging units. Dairy wastewater consists of a combination of water, solids, fats and cleaning agents. The volume and characteristics of dairy wastewater can vary depending on factors such as the type of dairy product produced, the processing methods used and the size of the farm.

Wastewater from the dairy industry must be properly treated before discharge, as it contains high levels of organic matter, nitrogen and phosphorus, which can contribute to water pollution.

- Treatment schemes for waste water from dairy

The main contaminants found in dairy industry wastewater are as follows:

- Organic matter: Dairy industry wastewater contains high levels of organic matter, including fats, proteins and lactose. This organic matter can contribute, if not properly treated, to elevated levels of biochemical oxygen demand (BOD) and chemical oxygen demand (COD) in receiving water bodies, which can reduce dissolved oxygen levels and harm aquatic life.

- Nutrients: Wastewater from the dairy industry is rich in nutrients such as nitrogen and phosphorus, which can contribute to the eutrophication of receiving water bodies. Eutrophication is the process by which excess nutrients cause rapid growth of algae and other aquatic plants, resulting in reduced water quality, oxygen depletion and fish kills.

- Pathogens: Dairy wastewater may contain bacteria, viruses and other microorganisms that can pose a risk to human health if ingested or inhaled. These pathogens can come from animal excrement, human excrement or contaminated equipment or surfaces in dairy processing facilities.

- Cleaning agents: Dairy processing facilities use a variety of cleaning agents, such as acids, alkalis and detergents, to clean and disinfect equipment and surfaces. These agents can be harmful to aquatic life and contribute to water pollution if not properly managed.

- Antibiotics and hormones: Wastewater from the dairy industry may contain residues of antibiotics and hormones used to treat or improve milk production in dairy cattle. These residues can contribute to the development of antibiotic-resistant bacteria and pose risks to human health and the environment.

- Textile

It’s important for textile factories to have an effective wastewater effluent treatment plan. Textile production processes are water-intensive and release harmful chemicals, dyes, acids and starches into the water. Textile effluent can cause significant water pollution that threatens human health and damages the environment.

The textile industry uses many industrial processes and substances that require physio-chemical treatment of the water used in those processes. Since textile wastewater is so harmful, textile plants must have an appropriate effluent treatment system to treat water effectively while saving time, space and money.

- Pharmaceutical industries

Treatment of discharges with emerging contaminants

In general, biological treatment processes are the most economical and common for treating wastewater, but the complexity of the components present in the wastewater from the pharmaceutical industry complicates the application of a specific treatment. Likewise, the presence of certain compounds such as antibiotics and disinfectants prevents the complete removal of contaminants through conventional biological treatments.

Advanced oxidation processes (AOP) + Biological treatments

Advanced oxidation processes (AOP) have proven to be highly effective for wastewater produced in the pharmaceutical industry, especially those containing micro contaminants or emerging contaminants.

This technology uses chemical oxidants to reduce chemical oxygen demand (COD) levels, as well as to break the bonds of these organic contaminant compounds and facilitate their biodegradability, thus saving reagents and reducing operational costs compared to attempting to oxidize all organic matter with reagents.

These treatments are based on physicochemical processes capable of producing profound changes in the chemical structure of contaminants, involving the generation and use of powerful transient species, mainly the hydroxyl radical (OH-), which is the strongest oxidizing agent after fluorine. Additionally, the generation of radicals occurs from oxygen, hydrogen peroxide, and supported catalysts, so the reaction byproducts are only water and carbon dioxide.

# WASTEWATER TREATMENT SCHEME – POWER PLANTS

Wastewater in thermal power plants comes from:

- Cooling tower blowdown

- Boiler blowdown

- Ash handling systems

- Oil-contaminated effluents

## Typical Treatment Flow (Scheme)

1. Screening → 2. Oil & Grease Trap → 3. Equalization Tank → 4. Neutralization → 5. Coagulation–Flocculation → 6. Clarification → 7. Filtration → 8. Advanced Treatment (if needed) → 9. Reuse/Discharge

## Step-wise Explanation

### 1. Screening

- Removes large debris (ash particles, solids)

- Protects downstream equipment

### 2. Oil & Grease Separator

- Removes oil from turbine and machinery runoff

- Uses gravity separation

### 3. Equalization Tank

- Maintains uniform flow and concentration

- Prevents shock loading

### 4. Neutralization

- Adjusts pH (acidic/alkaline wastewater)

- Uses acids (H₂SO₄) or alkalis (lime)

### 5. Coagulation–Flocculation

- Chemicals like alum or ferric chloride added

- Suspended particles form flocs

### 6. Clarifier (Sedimentation)

- Settles flocs and suspended solids

### 7. Filtration

- Sand or multimedia filters remove fine particles

### 8. Advanced Treatment (if required)

- Reverse Osmosis (RO)

- Ion Exchange

- Zero Liquid Discharge (ZLD) systems

### 9. Reuse/Discharge

- Cooling water reuse

- Safe discharge as per environmental norms

# Wastewater Treatment Scheme – Rice Mills (Agro-Based Industry)

## Characteristics of Rice Mill Effluent

- High organic load (BOD, COD)

- Suspended solids (husk, bran)

- Starch content

- Sometimes acidic nature

## Typical Treatment Flow (Scheme)

1. Screening → 2. Grit Chamber → 3. Equalization Tank → 4. Anaerobic Treatment → 5. Aerobic Treatment → 6. Secondary Clarifier → 7. Sludge Handling → 8. Disinfection → 9. Reuse/Discharge

## Step-wise Explanation

### 1. Screening

- Removes husk, straw, and large solids

### 2. Grit Chamber

- Removes sand and heavy particles

### 3. Equalization Tank

- Stabilizes flow and organic load

### 4. Anaerobic Treatment

- Common units: UASB reactor, anaerobic lagoon

- Breaks down organic matter without oxygen

- Produces biogas (can be used as energy)

### 5. Aerobic Treatment

- Activated Sludge Process / Aeration Tank

- Further reduces BOD and COD

### 6. Secondary Clarifier

- Settles biological sludge

- Clear water moves forward

### 7. Sludge Treatment

- Thickening and drying (sludge beds)

- Used as manure

### 8. Disinfection

- Chlorination or UV treatment

- Kills pathogens

### 9. Reuse/Discharge

- Irrigation use

- Safe discharge into water bodies